Arduino Circuit Design Report

Wiring Diagrams  •  Circuit Images  •  Code & Explanations

Complete Beginner's Guide  |  Platform: TinkerCAD  |  Board: Arduino Uno R3

# About This Report

This report documents four Arduino circuits created in TinkerCAD. Each section includes the actual circuit screenshot, a detailed wiring table, the complete code, and a beginner-friendly explanation of every line.

| Circuit 1 | RGB LED Color Cycling — PWM color mixing with analogWrite() |
| --- | --- |
| Circuit 2 | HC-SR04 Ultrasonic Distance Sensor — measuring distance with sound |
| Circuit 3 | Traffic Light System — real traffic light sequence with digitalWrite() |
| Circuit 4 | Servo + Ultrasonic — automated servo response to detected objects |

| 01 | RGB LED Color Cycling Pins 9,10,11 (PWM) — analogWrite() — #define |
| --- | --- |

## 1.1 Circuit Image

Circuit 1: RGB LED connected to Arduino Uno R3 (TinkerCAD simulation)

## 1.2 Overview

An RGB LED has three LEDs inside one package: Red, Green, Blue. By sending PWM values (0–255) through analogWrite(), each colour's brightness is controlled independently. This sketch cycles: Red → Green → Blue, each for 2 seconds.

## 1.3 Components

| Arduino Uno R3 | Main microcontroller |
| --- | --- |
| RGB LED (Common Cathode) | 4-legged LED: R, GND, G, B |
| 3 × 220 Ω Resistors | One per colour channel — protects LED from excess current |
| Wires | Connect Arduino pins to LED legs via resistors |

## 1.4 Wiring Diagram

| FROM (Component) | TO (Arduino) | Purpose |
| --- | --- | --- |
| RGB Pin R (Red leg) | Pin 9 → 220Ω resistor | PWM output for Red brightness |
| RGB Common Cathode | GND | Return path for all three channels |
| RGB Pin G (Green leg) | Pin 10 → 220Ω resistor | PWM output for Green brightness |
| RGB Pin B (Blue leg) | Pin 11 → 220Ω resistor | PWM output for Blue brightness |

⚠  Pins 9, 10, 11 are PWM-capable (marked ~ on the board). analogWrite() only works on PWM pins.

## 1.5 Complete Code

#define R 9 // Name pin 9 as R (Red) #define G 10 // Name pin 10 as G (Green) #define B 11 // Name pin 11 as B (Blue) void setup() { pinMode(R, OUTPUT); // Pin 9 → send signals OUT pinMode(G, OUTPUT); // Pin 10 → send signals OUT pinMode(B, OUTPUT); // Pin 11 → send signals OUT } void loop() { // ── RED phase ────────────────────────── analogWrite(R, 255); // Red = full brightness analogWrite(G, 0); // Green = OFF analogWrite(B, 0); // Blue = OFF delay(2000); // Hold for 2 seconds // ── GREEN phase ──────────────────────── analogWrite(R, 0); // Red = OFF analogWrite(G, 255); // Green = full brightness analogWrite(B, 0); // Blue = OFF delay(2000); // ── BLUE phase ───────────────────────── analogWrite(R, 0); // Red = OFF analogWrite(G, 0); // Green = OFF analogWrite(B, 255); // Blue = full brightness delay(2000); } // loop() restarts automatically

## 1.6 Code Explanation (Line by Line)

| #define R 9 | Creates a constant: anywhere we write R, Arduino reads 9. Makes code readable. |
| --- | --- |
| void setup() | Runs once at power-on. Used to configure pins. |
| pinMode(R, OUTPUT) | Declares pin 9 as an output — Arduino will send voltage on it. |
| void loop() | Runs forever in a cycle after setup() finishes. |
| analogWrite(R, 255) | PWM: sends ~5V (255/255) to Red pin → full brightness. |
| analogWrite(G, 0) | PWM: sends 0V to Green pin → completely off. |
| delay(2000) | Pauses execution for 2000 ms = 2 seconds. |
| // comment | Text after // is ignored by Arduino. Used to explain code to humans. |

| 02 | HC-SR04 Ultrasonic Distance Sensor Pins 9 (TRIG) & 10 (ECHO) — pulseIn() — Serial Monitor |
| --- | --- |

## 2.1 Circuit Image

Circuit 2: HC-SR04 ultrasonic sensor wired to Arduino Uno R3

## 2.2 Overview

The HC-SR04 works like sonar. The TRIG pin fires a short ultrasonic burst; the sensor waits for the echo. pulseIn() measures how long the echo signal stays HIGH. Dividing by the speed of sound gives distance in centimetres. Results stream to the Serial Monitor every 500 ms.

## 2.3 Components

| Arduino Uno R3 | Microcontroller |
| --- | --- |
| HC-SR04 Module | 4-pin ultrasonic sensor (VCC, TRIG, ECHO, GND) |
| 4 Wires | VCC, GND, TRIG, ECHO connections |

## 2.4 Wiring Diagram

| FROM (Component) | TO (Arduino) | Purpose |
| --- | --- | --- |
| HC-SR04 VCC | Arduino 5V | Power supply for the sensor |
| HC-SR04 GND | Arduino GND | Common ground |
| HC-SR04 TRIG | Arduino Pin 9 (OUTPUT) | Arduino triggers the ultrasonic pulse |
| HC-SR04 ECHO | Arduino Pin 10 (INPUT) | Sensor tells Arduino when echo returned |

## 2.5 Complete Code

#define TRIG 9 // Trigger: sends ultrasonic pulse #define ECHO 10 // Echo: receives reflected pulse long duration; // Time (µs) for sound to travel and return int distance; // Calculated distance in centimetres void setup() { Serial.begin(9600); // Open serial at 9600 baud pinMode(TRIG, OUTPUT); // TRIG → send pulse pinMode(ECHO, INPUT); // ECHO → read echo } void loop() { // Step 1 – ensure TRIG is LOW before firing digitalWrite(TRIG, LOW); delayMicroseconds(2); // Step 2 – send a 10-microsecond HIGH pulse digitalWrite(TRIG, HIGH); delayMicroseconds(10); digitalWrite(TRIG, LOW); // Step 3 – measure how long ECHO stays HIGH duration = pulseIn(ECHO, HIGH); // Step 4 – convert time to distance // Speed of sound = 0.034 cm/µs // Divide by 2: sound travels TO object AND back distance = duration * 0.034 / 2; // Step 5 – print result Serial.print("Distance: "); Serial.print(distance); Serial.println(" cm"); delay(500); // read every 0.5 seconds }

## 2.6 Code Explanation (Line by Line)

| long duration | 'long' holds large integers. Echo time in µs can exceed int's max value. |
| --- | --- |
| int distance | 'int' is enough for distances 0–400 cm. |
| Serial.begin(9600) | Starts the serial connection at 9600 bits/sec so data appears in the Serial Monitor. |
| delayMicroseconds(2) | Waits 2 µs — far shorter than delay(). Keeps the TRIG LOW reset clean. |
| delayMicroseconds(10) | Holds TRIG HIGH for exactly 10 µs — the spec-required trigger pulse width. |
| pulseIn(ECHO, HIGH) | Waits for ECHO to go HIGH, measures duration, returns value in microseconds. |
| duration * 0.034 / 2 | 0.034 cm/µs × time = round-trip distance. Halved = one-way (object distance). |
| Serial.println() | Like print() but appends a newline so each reading appears on its own row. |

| 03 | Traffic Light System Pins 8,9,10 — digitalWrite() — Breadboard — Timing |
| --- | --- |

## 3.1 Circuit Image

Circuit 3: Red/Yellow/Green LEDs on breadboard controlled by Arduino Uno R3

## 3.2 Overview

Three separate LEDs (Red, Yellow, Green) mounted on a breadboard simulate a real traffic light. Each LED has its own 220 Ω current-limiting resistor. The Arduino cycles through Green (5 s) → Yellow (2 s) → Red (5 s), exactly like a real intersection.

## 3.3 Components

| Arduino Uno R3 | Microcontroller |
| --- | --- |
| Breadboard | Prototype board — rows share electrical connection; no soldering needed |
| Red LED | Stop signal |
| Yellow LED | Caution / prepare to stop |
| Green LED | Go signal |
| 3 × 220 Ω Resistors | One per LED — limits current to safe ~20 mA |
| Wires | Arduino → breadboard connections |

## 3.4 Wiring Diagram

| FROM (Component) | TO (Arduino) | Purpose |
| --- | --- | --- |
| Green LED anode (+) | Arduino Pin 10 via 220Ω resistor | digitalWrite HIGH = Green ON |
| Yellow LED anode (+) | Arduino Pin 9 via 220Ω resistor | digitalWrite HIGH = Yellow ON |
| Red LED anode (+) | Arduino Pin 8 via 220Ω resistor | digitalWrite HIGH = Red ON |
| All LED cathodes (−) | Arduino GND (via breadboard −ve rail) | Common ground return |

Breadboard tip: long leg (anode +) faces the resistor; short leg (cathode −) goes to the GND rail.

## 3.5 Complete Code

#define RED 8 #define YELLOW 9 #define GREEN 10 void setup() { pinMode(RED, OUTPUT); pinMode(YELLOW, OUTPUT); pinMode(GREEN, OUTPUT); } void loop() { // ── Phase 1: GREEN ON (Go) ───────────────────── digitalWrite(GREEN, HIGH); // Green ON digitalWrite(YELLOW, LOW); // Yellow OFF digitalWrite(RED, LOW); // Red OFF delay(5000); // 5 seconds // ── Phase 2: YELLOW ON (Caution) ────────────── digitalWrite(GREEN, LOW); // Green OFF digitalWrite(YELLOW, HIGH); // Yellow ON digitalWrite(RED, LOW); // Red OFF delay(2000); // 2 seconds // ── Phase 3: RED ON (Stop) ──────────────────── digitalWrite(GREEN, LOW); // Green OFF digitalWrite(YELLOW, LOW); // Yellow OFF digitalWrite(RED, HIGH); // Red ON delay(5000); // 5 seconds }

## 3.6 Code Explanation (Line by Line)

| #define RED 8 | Aliases pin 8 as RED. Using names instead of numbers prevents pin confusion. |
| --- | --- |
| 3× pinMode OUTPUT | All three LED pins are configured as outputs in setup(). |
| digitalWrite(GREEN, HIGH) | Sets pin 10 to 5V — current flows → Green LED lights up. |
| digitalWrite(YELLOW, LOW) | Sets pin 9 to 0V — no current → Yellow LED stays off. |
| delay(5000) | Freezes the program for 5 seconds. Loop resumes at next line after delay. |
| Why turn others LOW? | Best practice: explicitly turn off every other LED each phase to avoid accidental multi-LED states. |
| delay(2000) | Yellow is shorter (2 s) — mimics real traffic light warning duration. |
| Loop auto-repeats | After Red's delay(5000), loop() jumps back to top → Green turns on again. |

| 04 | Servo Motor + Ultrasonic Sensor Servo.h library — if/else — Object detection — Automated response |
| --- | --- |

## 4.1 Circuit Image

Circuit 4: HC-SR04 ultrasonic sensor + SG90 servo motor on Arduino Uno R3

## 4.2 Overview

This circuit combines the HC-SR04 sensor with a servo motor. When an object comes within 20 cm, the servo rotates to 90°. When the path clears, it returns to 0°. This is the core principle behind automatic doors, robot obstacle avoidance, and security barriers.

## 4.3 Components

| Arduino Uno R3 | Microcontroller |
| --- | --- |
| HC-SR04 Module | Ultrasonic distance sensor |
| SG90 Servo Motor | 3-wire servo: Signal (orange), VCC (red), GND (brown) |
| Power Jack | External power — servo draws more current than USB alone can supply reliably |
| Wires | Multi-wire harness |

## 4.4 Wiring Diagram

| FROM (Component) | TO (Arduino) | Purpose |
| --- | --- | --- |
| HC-SR04 VCC | Arduino 5V | Sensor power |
| HC-SR04 GND | Arduino GND | Sensor ground |
| HC-SR04 TRIG | Arduino Pin 9 (OUTPUT) | Sends ultrasonic burst |
| HC-SR04 ECHO | Arduino Pin 10 (INPUT) | Receives reflected echo |
| Servo Signal (orange) | Arduino Pin 6 (PWM ~) | myServo.write() angle commands |
| Servo VCC (red) | Arduino 5V (or ext. power) | Motor power |
| Servo GND (brown) | Arduino GND | Motor ground |

⚠  Pin 6 must be a PWM pin (~). The Servo library sends a 50 Hz PWM signal to control angle.

## 4.5 Complete Code

#include <Servo.h> // Load Servo library #define TRIG 9 #define ECHO 10 Servo myServo; // Create servo object long duration; int distance; void setup() { Serial.begin(9600); pinMode(TRIG, OUTPUT); pinMode(ECHO, INPUT); myServo.attach(6); // Servo on pin 6 myServo.write(0); // Start position = 0° } void loop() { // ── Measure distance (same as Circuit 2) ────── digitalWrite(TRIG, LOW); delayMicroseconds(2); digitalWrite(TRIG, HIGH); delayMicroseconds(10); digitalWrite(TRIG, LOW); duration = pulseIn(ECHO, HIGH); distance = duration * 0.034 / 2; // ── Servo decision based on distance ────────── if (distance < 20) // Object closer than 20 cm? { myServo.write(90); // YES → rotate to 90° } else // Object far or none? { myServo.write(0); // NO → return to 0° } Serial.print("Distance: "); Serial.print(distance); Serial.println(" cm"); delay(100); }

## 4.6 Code Explanation (Line by Line)

| #include <Servo.h> | Imports the built-in Servo library. Without this, myServo commands won't compile. |
| --- | --- |
| Servo myServo | Declares a Servo object. Think of it as naming your servo 'myServo'. |
| myServo.attach(6) | Links myServo to pin 6. The library now sends PWM signals on that pin. |
| myServo.write(0) | Moves servo to 0 degrees. Range is 0–180. Call in setup() to set start position. |
| if (distance < 20) | Conditional check: is the measured distance less than 20 cm? |
| myServo.write(90) | If YES: rotate servo to 90° (barrier opens / door opens). |
| else { myServo.write(0) } | If NO: keep servo at 0° (closed position). |
| delay(100) | 100 ms pause stabilises sensor readings without making the servo feel sluggish. |

## 4.7 if / else — How Decisions Work in Arduino

if ( condition ) // Is this true? { // Run this code when TRUE } else { // Run this code when FALSE } // Real example from this circuit: if (distance < 20) // Is something within 20 cm? { myServo.write(90); // TRUE → open barrier } else { myServo.write(0); // FALSE → close barrier }

# Summary: All 4 Circuits at a Glance

| # | Circuit | Pins | Key Function | New Concept | Output |
| --- | --- | --- | --- | --- | --- |
| 01 | RGB LED | 9,10,11 (PWM) | Colour mixing | analogWrite() 0–255 | LED changes colour |
| 02 | Ultrasonic | 9 TRIG, 10 ECHO | Distance measure | pulseIn(), Serial | Prints cm to monitor |
| 03 | Traffic Light | 8,9,10 (Digital) | LED sequencing | digitalWrite() + timing | 3-phase light cycle |
| 04 | Servo+Sensor | 6 Servo,9,10 | Object detection | Servo.h, if/else | Motor reacts to distance |

# Beginner Tips & Common Mistakes

| Always add resistors to LEDs | Without a resistor, too much current flows — the LED burns out instantly. |
| --- | --- |
| Only PWM pins for analogWrite() | Pins marked ~ on Uno: 3,5,6,9,10,11. Plain digital pins will not work. |
| GND must be connected | Every component needs a GND wire back to Arduino or it won't function. |
| Serial.begin() before print() | Forgetting Serial.begin(9600) in setup() means nothing appears in the monitor. |
| delay() blocks everything | While waiting, Arduino ignores all other inputs. For complex timing use millis(). |
| #include before any library call | Servo.h (or any library) must be #included at the top of the sketch. |
| Check pin numbers carefully | Swapping a pin number between code and wiring is the most common beginner mistake. |
| Long leg = anode (+) | On any LED, the longer leg is + (anode). Short leg is − (cathode) → GND. |

— End of Report —