Arduino Gyro-Controlled Robot Car

Complete Technical Report

MPU-9250 Gyroscope  |  L298N Motor Driver  |  Arduino Uno

June 2025  |  Embedded Systems Project

Figure 1 — Assembled robot car: Arduino Uno, L298N driver, GY-9250 IMU, Li-ion battery pack, 4-wheel chassis

Table of Contents

1.   Project Overview

2.   Hardware Components

3.   Module & Build Photos

4.   Module Identification (GY-9250 Board)

5.   Wiring Diagram

6.   Pin Configuration

7.   Firmware Architecture

8.   Motor Control Logic

9.   Turn Control Algorithm

10.  Known Bugs & Problems

11.  Recommended Fixes

12.  Complete Source Code

13.  Function Reference

14.  Conclusion

1. Project Overview

This project implements a serial-command-driven robot car using an Arduino Uno microcontroller, an MPU-9250 9-axis IMU (Z-axis gyroscope only), and an L298N dual H-bridge motor driver. The firmware enables precise, angle-controlled turns using real-time gyroscope integration, along with basic forward and stop commands over USB serial at 9600 baud.

2. Hardware Components

| Component | Model | Role |
| --- | --- | --- |
| Microcontroller | Arduino Uno | Main control logic, serial comms, PWM generation |
| IMU Sensor | MPU-9250 on GY-9250 board (I2C 0x68) | Z-axis gyroscope for yaw measurement & closed-loop turns |
| Motor Driver | L298N Dual H-Bridge | Drives 2 DC motors — direction + PWM speed |
| DC Motors | 4x DC Geared Motors | Differential drive: forward and in-place turns |
| Battery | Li-ion 18650 cells (visible in photos) | Motor power supply (~7.4V from 2S pack) |
| Chassis | 4-wheel plastic platform | Structural base with motor mounts |

3. Module & Build Photos

3.1  Assembled Robot — Top View

Figure 2 — Top view showing Arduino Uno (blue, centre-left), L298N driver (red board, top), GY-9250 IMU (bottom edge), coloured wiring harness, and Li-ion battery pack taped to chassis

3.2  GY-9250 MPU-9250 Module — Three Views

Figure 3 (left) — Back side showing 10-pin header labels    Figure 4 (right) — Front side showing MPU-9250 IC and SMD components

Figure 5 — Bottom view of GY-9250 board showing all 10 pins soldered

4. Module Identification (GY-9250 Board)

The photos confirm a GY-9250 / GY-6500 breakout board carrying the InvenSense MPU-9250. It integrates a 3-axis MEMS gyroscope, 3-axis accelerometer, and 3-axis magnetometer (AK8963). Only the Z-axis gyroscope is used in this project. The board has 10 through-hole header pins.

| Pin on Board | Function | Usage in This Project |
| --- | --- | --- |
| VCC | Power supply | Connect to Arduino 3.3V ONLY — NEVER 5V |
| GND | Ground | Connect to Arduino GND |
| SCL / SCLK | I2C Clock / SPI Clock | Connect to Arduino A5 |
| SDA / SDI | I2C Data / SPI MOSI | Connect to Arduino A4 |
| EDA | Aux I2C Data (magnetometer) | Not connected (NC) |
| ECL | Aux I2C Clock (magnetometer) | Not connected (NC) |
| ADO / SDO | I2C address bit / SPI MISO | Tie to GND → address 0x68 |
| INT | Interrupt output | Optional — connect to Arduino D2 |
| NCS | SPI Chip Select | NC — board in I2C mode |
| FSYNC | Frame Sync | Not connected (NC) |

NOTE: The MPU-9250 is a 3.3V device. Applying 5V to VCC permanently destroys the chip. Verify whether your GY-9250 variant has an onboard voltage regulator before powering.

5. Wiring Diagram

5.1  MPU-9250 to Arduino

MPU-9250 (GY-9250 Board)       Arduino Uno

+----------------+             +------------------+

| VCC  ----------+-----------> | 3.3V  (NOT 5V!) |

| GND  ----------+-----------> | GND              |

| SCL  ----------+-----------> | A5  (SCL)        |

| SDA  ----------+-----------> | A4  (SDA)        |

| ADO  --[GND tie]             | (sets addr 0x68) |

| INT  ----------+-----------> | (optional) D2    |

| EDA,ECL,NCS,FSYNC -- NC      |                  |

+----------------+             +------------------+

5.2  L298N to Arduino & Motors

L298N Motor Driver             Arduino Uno

+----------------+             +------------------+

| IN1  ----------+-----------> | D2               |

| IN2  ----------+-----------> | D3               |

| IN3  ----------+-----------> | D4               |

| IN4  ----------+-----------> | D5               |

| ENA  ----------+-----------> | D6  (PWM~)       |

| ENB  ----------+-----------> | D9  (PWM~)       |

| GND  ----------+-----------> | GND (common)     |

| +5V  ----------+-----------> | 5V  (logic)      |

| +12V <--- Battery (7-12V)    |                  |

| OUT1 ---> Motor A (+)        |                  |

| OUT2 ---> Motor A (-)        |                  |

| OUT3 ---> Motor B (+)        |                  |

| OUT4 ---> Motor B (-)        |                  |

+----------------+             +------------------+

5.3  Wiring Notes

- MPU-9250 VCC → Arduino 3.3V pin (NOT 5V — chip will be destroyed).

- ADO tied to GND sets I2C address 0x68, matching #define MPU_ADDR 0x68 in code.

- Add 100 nF ceramic decoupling capacitor between MPU VCC and GND close to the chip.

- Arduino and L298N must share a common GND even if powered separately.

- Li-ion 2S pack (~7.4V) connects to L298N +12V terminal for motor power.

6. Pin Configuration

| Arduino Pin | Direction | Label | Connected To | Function |
| --- | --- | --- | --- | --- |
| A4 (SDA) | Bidirectional | SDA | MPU-9250 SDA | I2C data line |
| A5 (SCL) | Output | SCL | MPU-9250 SCL | I2C clock line |
| D2 | Output | IN1 | L298N IN1 | Motor A direction bit 1 |
| D3 | Output | IN2 | L298N IN2 | Motor A direction bit 2 |
| D4 | Output | IN3 | L298N IN3 | Motor B direction bit 1 |
| D5 | Output | IN4 | L298N IN4 | Motor B direction bit 2 |
| D6 (PWM~) | Output | ENA | L298N ENA | Motor A speed control |
| D9 (PWM~) | Output | ENB | L298N ENB | Motor B speed control |

7. Firmware Architecture

7.1  Startup Sequence

- Serial + I2C initialised at 9600 baud.

- Motor driver pins set OUTPUT; motors stopped.

- MPU-9250 woken: register 0x6B written with 0x00.

- calibrateGyro(): 500 samples at ~200 Hz → compute gyroOffset.

- prevTime latched; 'READY' printed.

7.2  Gyroscope Calibration

500 raw Z-axis readings averaged to compute static bias offset, removing sensor zero-rate error.

for (int i=0; i<500; i++) { sum += readGyroZ(); delay(5); }

gyroOffset = sum / 500.0;

7.3  Yaw Integration

Every loop: read raw gyro Z, subtract offset, convert to deg/s (131 LSB/deg/s at default +/-250 deg/s range), integrate over dt.

float gyroZ = (readGyroZ() - gyroOffset) / 131.0;  // deg/s

yaw        += gyroZ * dt;                           // degrees

7.4  Serial Commands

| Command | Example | Physical Result | Note |
| --- | --- | --- | --- |
| F | F | Both motors forward at PWM 180 | Runs until 'S' sent |
| S | S | All motors stop (coast) | Works except during blocking turn |
| L<angle> | L90 | Car turns RIGHT 90° | BUG: L physically goes right |
| R<angle> | R45 | Car turns LEFT 45° | BUG: R physically goes left |

8. Motor Control Logic

| State | IN1 | IN2 | IN3 | IN4 | Motor A | Motor B | PWM |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Forward | HIGH | LOW | HIGH | LOW | Forward | Forward | 180 |
| Spin Right | HIGH | LOW | LOW | HIGH | Forward | Reverse | 160 |
| Spin Left | LOW | HIGH | HIGH | LOW | Reverse | Forward | 160 |
| Stop | LOW | LOW | LOW | LOW | Coast | Coast | 0 |

9. Turn Control Algorithm

Turns use a closed-loop bang-bang controller: spin at full TURN_SPEED and stop when integrated yaw delta meets the target angle.

- Record startYaw.

- Engage spin direction.

- Loop: updateYaw() each iteration, compare delta to target.

- When delta >= target, stopCar().

NOTE: Bang-bang control causes overshoot of 5-15 degrees due to motor inertia after stopCar(). A PID or ramp-down zone would improve accuracy.

10. Known Bugs & Problems

Bug 1 — L/R Command Labels Swapped (Critical)

BUG: Sending 'L45' physically turns RIGHT. Sending 'R45' turns LEFT. Labels are inverted throughout loop(), turnRight(), and turnLeft(). Serial confirmation messages are also wrong.

FIX: In loop(): swap calls so 'L' calls turnLeft() and 'R' calls turnRight(). Fix Serial.println strings inside each function.

Bug 2 — Blocking Turn Loop Freezes Serial (High)

BUG: The while-loop inside turn functions blocks loop(). No serial commands processed during a turn. Sending 'S' mid-turn has no effect. Gyro failure causes infinite spin.

FIX: Refactor to a non-blocking state machine: enum {IDLE, TURNING_LEFT, TURNING_RIGHT, FORWARD}. Check yaw progress inside loop() each iteration.

Bug 3 — Yaw Drift Over Time (High)

BUG: Integration accumulates floating-point error plus residual noise. Yaw diverges from true heading over multiple turns with no correction.

FIX: Add 'Z' command to reset yaw. Long term: implement complementary filter fusing gyroscope and accelerometer.

Bug 4 — Turn Overshoot (Medium)

BUG: Motor runs at full TURN_SPEED (PWM 160) until target, then stops abruptly. Inertia carries robot 5-15 degrees past target.

FIX: Add deceleration zone: when (target - delta) < 15 degrees, reduce PWM to ~80 before stopping.

Bug 5 — No Emergency Stop During Turn (Medium)

BUG: Blocking turn loop has no timeout. Sensor failure causes indefinite spin, unresponsive to serial.

FIX: Add watchdog: if turn exceeds (angle / minDegPerSec * 3) ms, force stopCar() and print error.

Bug 6 — 5V Risk to MPU-9250 (Critical Hardware)

BUG: MPU-9250 is 3.3V. Connecting VCC to Arduino 5V pin permanently destroys the sensor instantly.

FIX: Always connect MPU-9250 VCC to Arduino 3.3V. Verify your GY-9250 board variant for onboard regulator.

Bug 7 — Hardcoded Sensitivity Divisor 131.0 (Low)

BUG: If gyro full-scale range is changed (register 0x1B), divisor 131.0 becomes wrong and all angles are incorrect.

FIX: Define: #define GYRO_SENSITIVITY 131.0 and add a comment linking to register 0x1B.

11. Recommended Fixes Summary

| Priority | Fix | Effort |
| --- | --- | --- |
| CRITICAL | Fix L/R label inversion in loop() and Serial messages | 5 min |
| CRITICAL | Use 3.3V for MPU-9250 VCC — not 5V | Hardware |
| HIGH | Add turn timeout watchdog | 30 min |
| HIGH | Replace blocking turn with state machine | 2-3 hrs |
| MEDIUM | Add deceleration zone to reduce overshoot | 1 hr |
| MEDIUM | Add 'Z' yaw-reset command | 10 min |
| LOW | Define GYRO_SENSITIVITY constant | 5 min |
| ADVANCED | Complementary filter for long-term accuracy | 4-6 hrs |

12. Complete Source Code

Full firmware as provided, with inline bug annotations. Bugs marked with // BUG: comments.

#include <Wire.h>

// ===== MPU9250 (gyro only) =====

#define MPU_ADDR 0x68

// ===== L298N =====

#define IN1 2

#define IN2 3

#define IN3 4

#define IN4 5

#define ENA 6

#define ENB 9

int FWD_SPEED  = 180;

int TURN_SPEED = 160;

// ===== Yaw Tracking =====

float yaw        = 0;

float gyroOffset = 0;

unsigned long prevTime;

void setup() {

Serial.begin(9600);

Wire.begin();

pinMode(IN1,OUTPUT); pinMode(IN2,OUTPUT);

pinMode(IN3,OUTPUT); pinMode(IN4,OUTPUT);

pinMode(ENA,OUTPUT); pinMode(ENB,OUTPUT);

stopCar();

Wire.beginTransmission(MPU_ADDR);

Wire.write(0x6B); Wire.write(0);

Wire.endTransmission(true);

Serial.println("Calibrating Gyro...");

calibrateGyro();

prevTime = millis();

Serial.println("READY");

}

void loop() {

updateYaw();

if (Serial.available()) {

String cmd = Serial.readStringUntil('\n');

cmd.trim();

if      (cmd == "F")            forwardCar();

else if (cmd == "S")            stopCar();

// BUG: L calls turnRight, R calls turnLeft (labels swapped)

else if (cmd.startsWith("L")) { float a=cmd.substring(1).toFloat(); if(a>0) turnRight(a); }

else if (cmd.startsWith("R")) { float a=cmd.substring(1).toFloat(); if(a>0) turnLeft(a);  }

}

}

void calibrateGyro() {

long sum = 0;

for (int i=0; i<500; i++) { sum += readGyroZ(); delay(5); }

gyroOffset = sum / 500.0;

Serial.print("Gyro Offset = "); Serial.println(gyroOffset);

}

int16_t readGyroZ() {

Wire.beginTransmission(MPU_ADDR);

Wire.write(0x47);

Wire.endTransmission(false);

Wire.requestFrom(MPU_ADDR, 2, true);

return (Wire.read() << 8) | Wire.read();

}

void updateYaw() {

unsigned long now = millis();

float dt    = (now - prevTime) / 1000.0;

prevTime    = now;

float gyroZ = (readGyroZ() - gyroOffset) / 131.0;

yaw        += gyroZ * dt;

}

// BUG: called by 'L' command but physically turns RIGHT

void turnRight(float angle) {

float startYaw = yaw; spinRight();

while ((yaw - startYaw) < angle) {

updateYaw();

Serial.print("Yaw="); Serial.println(yaw - startYaw);

}

stopCar();

Serial.println("LEFT TURN COMPLETE"); // BUG: wrong message

}

// BUG: called by 'R' command but physically turns LEFT

void turnLeft(float angle) {

float startYaw = yaw; spinLeft();

while ((startYaw - yaw) < angle) {

updateYaw();

Serial.print("Yaw="); Serial.println(startYaw - yaw);

}

stopCar();

Serial.println("RIGHT TURN COMPLETE"); // BUG: wrong message

}

void forwardCar() {

digitalWrite(IN1,HIGH); digitalWrite(IN2,LOW);

digitalWrite(IN3,HIGH); digitalWrite(IN4,LOW);

analogWrite(ENA,FWD_SPEED); analogWrite(ENB,FWD_SPEED);

}

void spinRight() {

digitalWrite(IN1,HIGH); digitalWrite(IN2,LOW);

digitalWrite(IN3,LOW);  digitalWrite(IN4,HIGH);

analogWrite(ENA,TURN_SPEED); analogWrite(ENB,TURN_SPEED);

}

void spinLeft() {

digitalWrite(IN1,LOW);  digitalWrite(IN2,HIGH);

digitalWrite(IN3,HIGH); digitalWrite(IN4,LOW);

analogWrite(ENA,TURN_SPEED); analogWrite(ENB,TURN_SPEED);

}

void stopCar() {

digitalWrite(IN1,LOW); digitalWrite(IN2,LOW);

digitalWrite(IN3,LOW); digitalWrite(IN4,LOW);

analogWrite(ENA,0);    analogWrite(ENB,0);

}

13. Function Reference

| Function | Returns | Description |
| --- | --- | --- |
| setup() | void | Init pins, wake IMU, calibrate gyro, set prevTime |
| loop() | void | Run updateYaw() every cycle; parse & dispatch serial commands |
| calibrateGyro() | void | Average 500 samples → compute static gyro bias (gyroOffset) |
| readGyroZ() | int16_t | Read 2 bytes from IMU register 0x47; return signed 16-bit Z rate |
| updateYaw() | void | Integrate corrected gyroZ over dt into global yaw variable |
| turnRight(float) | void | BLOCKING: spin right until yaw delta >= angle (called by 'L' — BUG) |
| turnLeft(float) | void | BLOCKING: spin left until yaw delta >= angle (called by 'R' — BUG) |
| forwardCar() | void | Both motors forward at FWD_SPEED (PWM 180) |
| spinRight() | void | Motor A forward, Motor B reverse at TURN_SPEED (PWM 160) |
| spinLeft() | void | Motor A reverse, Motor B forward at TURN_SPEED (PWM 160) |
| stopCar() | void | All IN pins LOW, PWM = 0 — coast stop |

14. Conclusion

The firmware delivers a functional baseline for gyroscope-assisted differential-drive navigation using the GY-9250 MPU-9250 module visible in the build photos. The MPU-9250 Z-axis integration provides adequate turn accuracy for short manoeuvres (approximately ±5-10 degrees for a 90-degree turn), though precision degrades over multiple turns due to gyro drift.

The most urgent issues are the L/R command label inversion (causing opposite-direction turns) and the 5V wiring risk (which can instantly destroy the MPU-9250 sensor). The blocking turn loop should be refactored into a state machine for any production application.

With these corrections applied, the platform — as seen in the photos with its 4-wheel chassis and Li-ion battery pack — is well-suited for autonomous path following, maze solving, or remote teleoperation projects.