BUILD YOUR OWN

4-WHEEL MANUAL & AUTONOMOUS ROBOT CAR

Complete Technical Report  |  Build Photos  |  Full Code Reference

Figure 1 — The completed 4-wheel robot car with Arduino UNO, L298N motor driver, and USB laptop connection

Arduino UNO R3 + L298N Motor Driver + 4× DC Gear Motors USB Laptop Control ¦ Arrow Keys ¦ Python & Processing ¦ 11 Commands ¦ 3 Speed Levels Semi-Autonomous ¦ Auto Sequences ¦ Phase Roadmap to Full Autonomy

# Section 1 — Project Overview

This is the complete technical report and code reference for a 4-wheel Arduino robot car built from scratch. Every stage is documented — from components and wiring to three complete control programs with all code explained line-by-line. All photos are from the actual build.

## 1.1  Your Actual Build — Photo Gallery

The following photos document the complete build progression from early prototype through final assembly.

| Fig 2 — Early prototype: 2-wheel prototype on acrylic chassis with L298N (red board, top) | Fig 3 — Final version: Arduino UNO + L298N mounted on chassis, USB cable connected |
| --- | --- |

| Fig 4 — Top-down view: Arduino UNO (blue, bottom), L298N driver (red, top), coloured wiring | Fig 5 — Wooden chassis variant: L298N centered, yellow TT motors on all 4 corners |
| --- | --- |

## 1.2  System Architecture

| Layer | Component | Role |
| --- | --- | --- |
| Brain | Arduino UNO R3 | Reads keyboard commands, controls all output pins |
| Muscle | L298N Motor Driver | H-bridge chip — takes weak Arduino signals, drives motors with battery power |
| Power (Motors) | Battery Pack 6V–9V | Heavy current for motors — 4×AA or 2×18650 |
| Power (Brain) | Laptop USB Cable | Powers Arduino AND carries all keyboard commands |
| Communication | USB Serial 9600 baud | Road that carries F/B/L/R/S commands laptop→Arduino |
| Legs | 4× TT DC Gear Motors | Physical actuators that spin and move the car |

# Section 2 — Components & Hardware

| Fig 6 — Motor close-up: 4× TT gear motors with red/black wiring visible, mounted to acrylic chassis | Fig 7 — Underside view: 4 motors exposed with colour-coded wires (black/yellow/red/green) |
| --- | --- |

Fig 8 — Build in progress: 4 motors mounted on chassis, wires being routed, battery connector visible

## 2.1  Full Parts List

| Component | Qty | Function | Analogy |
| --- | --- | --- | --- |
| Arduino UNO R3 | 1 | Microcontroller — runs code, controls all pins | Human brain |
| L298N Motor Driver | 1 | H-bridge — controls direction & speed of 2 motor channels | Strong worker |
| TT DC Gear Motor (yellow) | 4 | 3V–6V gear motor — spins the wheels | Legs/feet |
| 65mm Rubber Wheel | 4 | Fits motor shaft — provides grip on surface | Shoes |
| 4WD Robot Chassis | 1 | Acrylic/wood/metal frame — holds everything | Skeleton |
| Battery Pack (6V–9V) | 1 | Powers motors via L298N — 4×AA or Li-Ion | Fuel |
| USB Type-B Cable | 1 | Powers Arduino + carries serial commands | Telephone line |
| Jumper Wires (M-M) | 20+ | All electrical connections | Roads |
| HC-05 Bluetooth* | 1 | Future upgrade — wireless phone control | Wireless remote |
| HC-SR04 Ultrasonic* | 1 | Future upgrade — obstacle detection | Eyes |
| TCRT5000 IR Sensor* | 2 | Future upgrade — line following | Floor sensors |

* = Optional, for future upgrade phases only.

# Section 3 — Wiring & Pin Connections

## 3.1  Arduino → L298N Pin Table

| Arduino Pin | L298N Pin | Color | Purpose |
| --- | --- | --- | --- |
| D2 (PWM) | ENA | Blue | Enable Left motors — PWM controls left motor speed |
| D3 | IN1 | Blue | Left motor direction signal 1 |
| D4 | IN2 | Blue | Left motor direction signal 2 |
| D5 | IN3 | Blue | Right motor direction signal 1 |
| D6 | IN4 | Blue | Right motor direction signal 2 |
| D9 (PWM) | ENB | Blue | Enable Right motors — PWM controls right motor speed |
| GND | GND | Black | CRITICAL — common ground. Without this NOTHING works. |
| 5V (optional) | 5V OUT | Red | Power Arduino from L298N regulator (battery ≤12V only) |

## 3.2  Battery → L298N & Motors

| From | To | Color | Purpose |
| --- | --- | --- | --- |
| Battery + (positive) | VIN / 12V terminal | Red | Main motor power supply (use 6V–9V) |
| Battery − (negative) | GND terminal | Black | Battery ground reference |
| L298N OUT1 + OUT2 | Left Motors ×2 | Green | Both left motors wired in parallel |
| L298N OUT3 + OUT4 | Right Motors ×2 | Yellow | Both right motors wired in parallel |

CRITICAL — The Common Ground Wire Connect: Arduino GND pin → L298N GND terminal (black jumper wire). This makes both power supplies (USB and battery) share the same voltage reference. Without this wire: L298N cannot understand any Arduino signals. Car will not move at all. This is the single most common beginner mistake in this entire build.

Dual Power Supply — Simple Explanation USB Cable from Laptop: Powers Arduino (5V) AND carries keyboard commands. Connected to laptop. Battery Pack (6V–9V): Powers L298N + all 4 motors. NOT connected to laptop. The two supplies are completely separate. The GND wire bridges them so signals work. Never connect 9V+ battery directly to Arduino — it will permanently damage it.

# Section 4 — Movement Logic & All Commands

## 4.1  L298N Truth Table

| IN1 | IN2 | Motor Result | Explanation |
| --- | --- | --- | --- |
| HIGH | LOW | Spin Forward | Positive current flows through motor in forward direction |
| LOW | HIGH | Spin Backward | Current reversed — motor spins backward |
| LOW | LOW | Coast (Stop) | No current — motor spins down under inertia |
| HIGH | HIGH | Brake (Stop) | Both terminals connected — motor stops hard |

## 4.2  All 11 Commands Explained

| Key | Command | Left Motor | Right Motor | What Happens |
| --- | --- | --- | --- | --- |
| F / ↑ | Forward | FORWARD | FORWARD | Both sides push — car goes straight ahead |
| B / ↓ | Backward | BACKWARD | BACKWARD | Both sides reverse — car goes straight back |
| L / ← | Turn Left | STOP | FORWARD | Right pushes, left anchored — car curves left |
| R / → | Turn Right | FORWARD | STOP | Left pushes, right anchored — car curves right |
| S / SPACE | Stop | STOP | STOP | All motors off — car halts immediately |
| G | Rotate Left | BACKWARD | FORWARD | Opposite dirs — spins counterclockwise in place |
| I | Rotate Right | FORWARD | BACKWARD | Opposite dirs — spins clockwise in place |
| A | Auto Fwd+Left | Sequence | Sequence | Fwd 2s → settle → TurnL → Fwd 3s → Stop |
| D | Auto Fwd+Right | Sequence | Sequence | Fwd 2s → settle → TurnR → Fwd 3s → Stop |
| Z | Auto Bwd+Left | Sequence | Sequence | Bwd 2s → settle → TurnL → Bwd 3s → Stop |
| X | Auto Bwd+Right | Sequence | Sequence | Bwd 2s → settle → TurnR → Bwd 3s → Stop |

## 4.3  Speed Levels

| Key | Level | analogWrite Value | Power % | Best For |
| --- | --- | --- | --- | --- |
| 1 | Slow | 60 | 24% | Tight spaces, precision, first-time testing |
| 2 | Medium | 130 | 51% | Normal indoor driving |
| (default) | Fast | 190–200 | 74–78% | Standard forward/backward |
| 3 | Fast | 200 | 78% | Open spaces, performance runs |
| — | Full | 255 | 100% | Maximum — harder to control on smooth floors |

# Section 5 — Three Laptop Control Methods

| Feature | Serial Monitor | Processing IDE | Python VS Code |
| --- | --- | --- | --- |
| Difficulty | Very Easy | Easy | Moderate |
| Hold Key = Move | No | Yes | Yes |
| Release Key = Stop | No | Yes — instant | Yes — instant |
| Arrow Key Support | No | Yes | Yes |
| Speed Keys (1/2/3) | Yes (type it) | No | Yes — real-time |
| Auto Sequences | Yes (type A/D/Z/X) | Yes (letter keys) | Yes — all 11 |
| Rotate G/I | Yes | Yes | Yes |
| Extra Software | None (Arduino IDE) | Processing IDE (free) | Python + pyserial + keyboard |
| Best For | Testing/Debug | RC-style driving | Advanced use |

## 5.1  Serial Monitor — Setup

- Upload Arduino code. Open Arduino IDE.

- Tools → Serial Monitor. Set baud to 9600 at bottom dropdown.

- Type F / B / L / R / S / G / I / A / D / Z / X / 1 / 2 / 3, press Enter.

- Car acts on command. Does NOT auto-stop on key release.

## 5.2  Processing IDE — Setup

- Download Processing from processing.org/download (free, ~100MB).

- Open Processing. Paste the Processing code (Section 9). Change "COM5" to your port.

- CLOSE Arduino Serial Monitor first — only one app can use the COM port.

- Click Run ▶. Dark window appears. CLICK INSIDE IT to activate.

- Hold Arrow Keys to move. Release any key — car stops instantly.

## 5.3  Python VS Code — Setup

- Install Python 3 from python.org. Open terminal.

- Run: pip install pyserial keyboard

- Open Python script. Change COM_PORT = "COM5" to your port.

- Run: python rc_car_controller.py

- Hold Arrow Keys to drive. Keys 1/2/3 change speed. ESC to quit.

# Section 6 — Problems, Causes & Solutions

### Problem 1: Motors not moving at all

PROBLEM: Car Completely Dead — No Motors Move Cause A: ENA/ENB jumper caps missing on L298N → Add caps or wire to Arduino PWM pins 6 and 9. Cause B: Arduino GND not connected to L298N GND → Add black wire: Arduino GND → L298N GND. Cause C: Battery dead or below 5V → Replace or recharge. Minimum 6V needed. Cause D: Wrong COM port → Check Arduino IDE Tools → Port. Re-upload sketch.

### Problem 2: One motor spins wrong direction

PROBLEM: Car Reverses or Turns Opposite to Expected Cause: Motor wires connected with reversed polarity at L298N output terminals. Fix (hardware): Swap the two wires of the affected motor at L298N output. Left wrong? Swap OUT1 and OUT2. Right wrong? Swap OUT3 and OUT4. Fix (software): In the motor function, swap HIGH and LOW for IN1/IN2 or IN3/IN4.

### Problem 3: Car drifts left or right

PROBLEM: Car Curves Instead of Going Straight Forward Cause: Budget DC motors have manufacturing tolerance — identical code produces slightly different speeds. Fix: Use separate L_SPEED and R_SPEED constants. Tune in steps of 5. Car drifts RIGHT? → Lower L_SPEED by 5 (e.g. 190→185). Re-upload. Test. Car drifts LEFT? → Lower R_SPEED by 5 (e.g. 200→195). Re-upload. Test. Target: car travels straight for 2+ meters on a flat surface.

### Problem 4: No COM port detected

PROBLEM: Arduino Not Visible in Arduino IDE Port List Cause A: CH340 driver not installed → Search "CH340 driver" and install for your OS. Cause B: Charge-only USB cable (no data lines) → Try a different cable. Cause C: Faulty USB port → Try a different port. Avoid USB hubs. Verify: Arduino PWR LED (red) should glow when USB is connected.

### Problem 5: Garbled text in Serial Monitor

PROBLEM: Serial Monitor Shows ??? or Random Characters Cause: Baud rate mismatch between Arduino code and Serial Monitor setting. Fix: In Arduino code: Serial.begin(9600) — must be 9600. Fix: In Serial Monitor: bottom dropdown must also be 9600 baud. Both must match exactly. After changing, close and reopen Serial Monitor.

### Problem 6: Car slides into turns (auto sequences)

PROBLEM: Car Carries Momentum Into Turns — Slides Through Cause: Early code called moveForward() then immediately turnLeft() — car was still moving. Fix (applied in Phase 3): Add stopCar() + delay(400) between every drive and turn phase. moveForward(); delay(2000); // drive stopCar(); delay(400); // SETTLE — car fully stops first turnLeft(); delay(700); // now turns cleanly from rest

### Problem 7: Processing cannot open COM port

PROBLEM: Processing Crashes — Could Not Open Port Cause: Arduino Serial Monitor is still open and holding the port exclusively. Fix: Close Arduino Serial Monitor window completely. Then run Processing. If still fails: unplug and replug Arduino USB cable, then run Processing again.

# Section 7 — Autonomy Phases & Upgrade Roadmap

| PHASE 1 | Manual — Serial Monitor Type commands in Arduino IDE Serial Monitor. Tests all wiring and motor logic. No auto-stop. | COMPLETE |
| --- | --- | --- |

| PHASE 2 | Hold-Key — Processing IDE Processing detects key press/release. Car moves while held, stops on release. Arrow keys added. | COMPLETE |
| --- | --- | --- |

| PHASE 3 | Auto Sequences + Drift Fix Added A/D/Z/X auto sequences. Fixed drift with L_SPEED/R_SPEED. Fixed slides with settle pauses. | COMPLETE |
| --- | --- | --- |

| PHASE 4 | Python + Speed Control Full Python controller. Speed levels 1/2/3. All 11 commands. Clean ESC exit. Console feedback. | COMPLETE |
| --- | --- | --- |

| PHASE 5 | Bluetooth — HC-05 Add HC-05 module. Phone app control. No USB cable needed. SoftwareSerial on pins 10/11. | PLANNED |
| --- | --- | --- |

| PHASE 6 | Obstacle Avoidance — HC-SR04 Ultrasonic sensor. Measure distance 10x/sec. Auto stop + turn if obstacle < 20cm. | PLANNED |
| --- | --- | --- |

| PHASE 7 | Line Following — TCRT5000 2 IR sensors on underside. Follows black line on white surface. Fully autonomous. | PLANNED |
| --- | --- | --- |

| PHASE 8 | WiFi + Camera — ESP32-CAM Browser control, live video stream. Full FPV rover experience. | PLANNED |
| --- | --- | --- |

## 7.2  Upgrade Parts Reference

| Upgrade | New Parts Needed | Difficulty | Key Benefit |
| --- | --- | --- | --- |
| Bluetooth | HC-05 module + phone app | ★★☆☆☆ Easy | Cut the cable — control wirelessly from phone |
| IR Remote | VS1838B + any TV remote | ★★☆☆☆ Easy | Use any TV remote as controller |
| Obstacle Avoidance | HC-SR04 ultrasonic | ★★★☆☆ Medium | Car detects and avoids obstacles automatically |
| Line Following | 2× TCRT5000 sensors | ★★★★☆ Hard | Fully autonomous — no human input needed |
| WiFi Control | NodeMCU ESP8266 | ★★★★☆ Hard | Browser-based control from any device |
| FPV Camera | ESP32-CAM + FTDI | ★★★★★ Advanced | Live video while remotely driving |

# Section 8 — Complete Arduino Code

How to Upload: Arduino IDE → Board: Arduino UNO → Tools → Port: your COM port → Click Upload (→) Wait for "Done uploading". Then open Serial Monitor (9600 baud) to verify "RC CAR READY" banner. CLOSE Serial Monitor before running Processing or Python controller.

rc_car_final.ino ¦ Arduino C++ ¦ Upload to: Arduino UNO R3 ¦ Baud: 9600

// ================================================================ // RC CAR — COMPLETE FINAL ARDUINO CODE // Board : Arduino UNO R3 // Driver : L298N Motor Driver // ---------------------------------------------------------------- // ALL COMMANDS (send via Serial / Processing / Python): // F/f = Forward B/b = Backward // L/l = Turn Left R/r = Turn Right // S/s = Stop G/g = Rotate Left (spin in place) // I/i = Rotate Right A/a = Auto Forward + Left // D/d = Auto Forward + Right // Z/z = Auto Backward + Left // X/x = Auto Backward + Right // 1 = Speed Slow(60) 2 = Medium(130) 3 = Fast(200) // ================================================================ // ── PIN DEFINITIONS ────────────────────────────────────────────── const int IN1 = 2; // Left Motor: Direction Pin 1 const int IN2 = 3; // Left Motor: Direction Pin 2 const int IN3 = 4; // Right Motor: Direction Pin 1 const int IN4 = 5; // Right Motor: Direction Pin 2 const int ENA = 6; // Left Motor: Enable/Speed (PWM) const int ENB = 9; // Right Motor: Enable/Speed (PWM) // ── SPEED TUNING ───────────────────────────────────────────────── // Drift RIGHT? -> Lower L_SPEED by 5 until straight // Drift LEFT? -> Lower R_SPEED by 5 until straight int L_SPEED = 190; // Left motor (0-255) int R_SPEED = 200; // Right motor (0-255) int TURN_SPEED = 160; // Turn/rotate speed (0-255) void setup() { pinMode(IN1,OUTPUT); pinMode(IN2,OUTPUT); pinMode(IN3,OUTPUT); pinMode(IN4,OUTPUT); pinMode(ENA,OUTPUT); pinMode(ENB,OUTPUT); Serial.begin(9600); Serial.println("=== RC CAR READY ==="); Serial.println("F=Fwd B=Bwd L=TurnL R=TurnR"); Serial.println("G=RotL I=RotR S=Stop"); Serial.println("A=AutoFL D=AutoFR Z=AutoBL X=AutoBR"); Serial.println("1=Slow 2=Med 3=Fast"); stopCar(); } void loop() { if (Serial.available() > 0) { char cmd = Serial.read(); switch (cmd) { case 'F': case 'f': moveForward(); break; case 'B': case 'b': moveBackward(); break; case 'L': case 'l': turnLeft(); break; case 'R': case 'r': turnRight(); break; case 'S': case 's': stopCar(); break; case 'G': case 'g': rotateLeft(); break; case 'I': case 'i': rotateRight(); break; case 'A': case 'a': autoForwardLeft(); break; case 'D': case 'd': autoForwardRight(); break; case 'Z': case 'z': autoBackwardLeft(); break; case 'X': case 'x': autoBackwardRight(); break; case '1': L_SPEED=60; R_SPEED=60; Serial.println("SPEED: SLOW"); break; case '2': L_SPEED=130; R_SPEED=130; Serial.println("SPEED: MEDIUM"); break; case '3': L_SPEED=190; R_SPEED=200; Serial.println("SPEED: FAST"); break; } } } // ── MOVEMENT FUNCTIONS ─────────────────────────────────────────── // FORWARD: IN1=HIGH IN2=LOW (left), IN3=HIGH IN4=LOW (right) void moveForward() { digitalWrite(IN1,HIGH); digitalWrite(IN2,LOW); digitalWrite(IN3,HIGH); digitalWrite(IN4,LOW); analogWrite(ENA,L_SPEED); analogWrite(ENB,R_SPEED); Serial.println(">> FORWARD"); } // BACKWARD: IN1=LOW IN2=HIGH (left), IN3=LOW IN4=HIGH (right) void moveBackward() { digitalWrite(IN1,LOW); digitalWrite(IN2,HIGH); digitalWrite(IN3,LOW); digitalWrite(IN4,HIGH); analogWrite(ENA,L_SPEED); analogWrite(ENB,R_SPEED); Serial.println(">> BACKWARD"); } // TURN LEFT: Left=STOP, Right=FORWARD void turnLeft() { digitalWrite(IN1,LOW); digitalWrite(IN2,LOW); digitalWrite(IN3,HIGH); digitalWrite(IN4,LOW); analogWrite(ENA,0); analogWrite(ENB,TURN_SPEED); Serial.println(">> TURN LEFT"); } // TURN RIGHT: Left=FORWARD, Right=STOP void turnRight() { digitalWrite(IN1,HIGH); digitalWrite(IN2,LOW); digitalWrite(IN3,LOW); digitalWrite(IN4,LOW); analogWrite(ENA,TURN_SPEED); analogWrite(ENB,0); Serial.println(">> TURN RIGHT"); } // STOP: All motors off void stopCar() { digitalWrite(IN1,LOW); digitalWrite(IN2,LOW); digitalWrite(IN3,LOW); digitalWrite(IN4,LOW); analogWrite(ENA,0); analogWrite(ENB,0); Serial.println(">> STOP"); } // ROTATE LEFT: Left=BACKWARD, Right=FORWARD (spin CCW in place) void rotateLeft() { digitalWrite(IN1,LOW); digitalWrite(IN2,HIGH); digitalWrite(IN3,HIGH); digitalWrite(IN4,LOW); analogWrite(ENA,TURN_SPEED); analogWrite(ENB,TURN_SPEED); Serial.println(">> ROTATE LEFT"); } // ROTATE RIGHT: Left=FORWARD, Right=BACKWARD (spin CW in place) void rotateRight() { digitalWrite(IN1,HIGH); digitalWrite(IN2,LOW); digitalWrite(IN3,LOW); digitalWrite(IN4,HIGH); analogWrite(ENA,TURN_SPEED); analogWrite(ENB,TURN_SPEED); Serial.println(">> ROTATE RIGHT"); } // ── AUTO SEQUENCES ─────────────────────────────────────────────── // Pattern: Move -> stopCar()+delay(400ms settle) -> Turn -> Move -> Stop // The 400ms SETTLE PAUSE is critical — prevents momentum carry into turn void autoForwardLeft() { Serial.println("AUTO: Fwd+Left"); moveForward(); delay(2000); stopCar(); delay(400); // settle pause turnLeft(); delay(700); stopCar(); delay(400); moveForward(); delay(3000); stopCar(); Serial.println("AUTO: Done"); } void autoForwardRight() { Serial.println("AUTO: Fwd+Right"); moveForward(); delay(2000); stopCar(); delay(400); turnRight(); delay(700); stopCar(); delay(400); moveForward(); delay(3000); stopCar(); Serial.println("AUTO: Done"); } void autoBackwardLeft() { Serial.println("AUTO: Bwd+Left"); moveBackward(); delay(2000); stopCar(); delay(400); turnLeft(); delay(700); stopCar(); delay(400); moveBackward(); delay(3000); stopCar(); Serial.println("AUTO: Done"); } void autoBackwardRight() { Serial.println("AUTO: Bwd+Right"); moveBackward(); delay(2000); stopCar(); delay(400); turnRight(); delay(700); stopCar(); delay(400); moveBackward(); delay(3000); stopCar(); Serial.println("AUTO: Done"); }

## 8.1  Arduino Code — Key Sections Explained

| Code Section | What It Does | Why It Matters |
| --- | --- | --- |
| PIN DEFINITIONS (top) | Maps variable names to Arduino pin numbers | Change one number = whole program updates. No hunting. |
| L_SPEED / R_SPEED | Separate speed values for left and right motors | Tune to fix drift — lower the side the car drifts toward |
| TURN_SPEED | Speed for turn and rotate commands | Lower = tighter turns. Too low = car barely moves. |
| Serial.begin(9600) | Opens USB communication channel | Without this, laptop and Arduino cannot talk at all |
| switch(cmd) | Routes each received character to a function | Add new command = add one new case line only |
| case "1"/"2"/"3" | Changes L_SPEED/R_SPEED at runtime | No re-upload needed to change speed while driving |
| analogWrite(ENA, speed) | Sets motor speed via PWM (0–255) | Higher value = faster motor — proportional control |
| digitalWrite(IN1, HIGH) | Sets direction pin to control motor spin | HIGH+LOW on IN1/IN2 = forward; LOW+HIGH = backward |
| stopCar() + delay(400) | Settle pause in every auto sequence | Car fully stops before turning — prevents momentum slide |

# Section 9 — Complete Processing Sketch

Install: processing.org/download (free ~100MB) → Paste code → Change COM5 → Click Run ▶ A dark window appears. CLICK INSIDE IT to make it active, then use arrow keys. Close Arduino Serial Monitor before running — they cannot share the COM port.

rc_car_controller.pde ¦ Processing Java ¦ Run in: Processing IDE ¦ Change "COM5" first

// ================================================================ // RC CAR — PROCESSING CONTROLLER // Language : Processing Java (processing.org/download — free) // ---------------------------------------------------------------- // SETUP: Change "COM5" to your actual Arduino port first! // Windows: COM3 / COM4 / COM5 (check Arduino IDE Tools>Port) // Mac: /dev/tty.usbmodem... // Linux: /dev/ttyUSB0 // ---------------------------------------------------------------- // HOW IT WORKS: // keyPressed() -> sends movement command to Arduino // keyReleased() -> sends 'S' (Stop) to Arduino instantly // RESULT: Hold key = move, Release key = stops IMMEDIATELY // ================================================================ import processing.serial.*; Serial myPort; String currentAction = "STOPPED"; void setup() { size(500, 280); textFont(createFont("Arial", 14)); // !! CHANGE "COM5" to your port !! myPort = new Serial(this, "COM5", 9600); } void draw() { background(25); // Title bar fill(100, 180, 255); textSize(18); textAlign(CENTER); text("RC CAR CONTROLLER", width/2, 35); // Click reminder fill(255, 220, 80); textSize(13); text("CLICK INSIDE THIS WINDOW FIRST, THEN DRIVE", width/2, 62); // Current action display fill(50, 220, 80); textSize(24); text(">> " + currentAction, width/2, 108); // Key guide fill(180); textSize(12); textAlign(LEFT); text("ARROW KEYS: UP=Forward DOWN=Backward", 28, 148); text(" LEFT=TurnLeft RIGHT=TurnRight", 28, 167); text("LETTERS: G=RotateLeft I=RotateRight", 28, 190); text(" A=AutoFwdL D=AutoFwdR", 28, 209); text(" Z=AutoBwdL X=AutoBwdR", 28, 228); text("SPACE = Emergency Stop", 28, 252); } // KEY PRESSED — send movement command ───────────────────────────── void keyPressed() { if (key == CODED) { if (keyCode == UP) { myPort.write('F'); currentAction = "FORWARD"; } if (keyCode == DOWN) { myPort.write('B'); currentAction = "BACKWARD"; } if (keyCode == LEFT) { myPort.write('L'); currentAction = "TURN LEFT"; } if (keyCode == RIGHT) { myPort.write('R'); currentAction = "TURN RIGHT"; } } else { char c = Character.toUpperCase(key); if (c == 'G') { myPort.write('G'); currentAction = "ROTATE LEFT"; } if (c == 'I') { myPort.write('I'); currentAction = "ROTATE RIGHT"; } if (c == 'A') { myPort.write('A'); currentAction = "AUTO: Fwd+Left"; } if (c == 'D') { myPort.write('D'); currentAction = "AUTO: Fwd+Right"; } if (c == 'Z') { myPort.write('Z'); currentAction = "AUTO: Bwd+Left"; } if (c == 'X') { myPort.write('X'); currentAction = "AUTO: Bwd+Right"; } if (c == ' ') { myPort.write('S'); currentAction = "STOPPED"; } } } // KEY RELEASED — send Stop instantly ────────────────────────────── void keyReleased() { if (key == CODED) { // Release any arrow key -> Stop car immediately if (keyCode==UP ¦¦ keyCode==DOWN ¦¦ keyCode==LEFT ¦¦ keyCode==RIGHT) { myPort.write('S'); currentAction = "STOPPED"; } } else { char c = Character.toUpperCase(key); if (c == 'G' ¦¦ c == 'I') { myPort.write('S'); currentAction = "STOPPED"; } // Note: A D Z X auto-sequences run to completion on Arduino // Do NOT send Stop on their release — that would interrupt them } }

## 9.1  Processing Code — Key Parts Explained

| Code Part | What It Does | Why It Matters |
| --- | --- | --- |
| import processing.serial.* | Loads built-in Serial library | Required to open and write to the COM port |
| new Serial(this, "COM5", 9600) | Opens COM port at 9600 baud | Change "COM5" to your port. Must match Arduino Serial.begin(9600). |
| void draw() | Runs 60 fps — draws the UI window | Shows current action text so you know what command is active |
| void keyPressed() | Fires once when key is first pressed | Sends movement command (F/B/L/R/G/I) to Arduino |
| void keyReleased() | Fires instantly when key is lifted | Sends "S" (Stop) — this creates hold-to-move / release-to-stop |
| key == CODED | Checks if it is a special key (arrow etc.) | Arrow keys are not regular chars — must check CODED first |
| Character.toUpperCase(key) | Converts key to uppercase | Accepts both lowercase and uppercase — same result |
| myPort.write("F") | Sends one character to Arduino over USB | One character triggers the full moveForward() on Arduino |
| A/D/Z/X not stopped on release | Auto sequences run to completion | Stopping mid-sequence would leave motors in undefined state |

# Section 10 — Complete Python Script

Install once: pip install pyserial keyboard ¦ Then change COM_PORT = "COM5" to your port pyserial: enables Python to send data over the USB serial port to Arduino. keyboard: detects real-time key hold state — essential for hold-to-move behavior. Run: python rc_car_controller.py

rc_car_controller.py ¦ Python 3 ¦ Run: python rc_car_controller.py ¦ Change COM_PORT first

# ================================================================ # RC CAR — PYTHON CONTROLLER (Full Featured) # Language : Python 3 ¦ Run in VS Code or any terminal # ---------------------------------------------------------------- # INSTALL LIBRARIES (run once in terminal): # pip install pyserial keyboard # ---------------------------------------------------------------- # CONTROLS: # Arrow Keys = Forward / Backward / Turn Left / Turn Right # Hold key = Car moves ¦ Release key = Car stops INSTANTLY # G = Rotate Left (spin in place counterclockwise) # I = Rotate Right (spin in place clockwise) # A = Auto: Forward then Left (full sequence) # D = Auto: Forward then Right (full sequence) # Z = Auto: Backward then Left (full sequence) # X = Auto: Backward then Right (full sequence) # 1 = Speed SLOW (analogWrite 60 = 24% power) # 2 = Speed MEDIUM (analogWrite 130 = 51% power) # 3 = Speed FAST (analogWrite 200 = 78% power) # SPACE = Emergency Stop # ESC = Quit (sends Stop before closing) # ================================================================ import serial, keyboard, time, sys # ── CONFIG ─────────────────────────────────────────────────────── # Change COM_PORT to your Arduino port: # Windows: "COM3" / "COM4" / "COM5" (check Device Manager) # Mac: "/dev/tty.usbmodem..." # Linux: "/dev/ttyUSB0" or "/dev/ttyACM0" COM_PORT = "COM5" BAUD_RATE = 9600 # ── CONNECT ────────────────────────────────────────────────────── print("=" * 42) print(" RC CAR PYTHON CONTROLLER") print("=" * 42) print(f"Connecting to {COM_PORT} at {BAUD_RATE} baud...") try: arduino = serial.Serial(COM_PORT, BAUD_RATE, timeout=0.1) time.sleep(2) # Wait for Arduino to reboot after serial open print(f"Connected!\n") except serial.SerialException as e: print(f"ERROR: Cannot connect to {COM_PORT}") print(f"Details: {e}") print("Fix: Update COM_PORT variable to your Arduino port") sys.exit(1) print("CONTROLS") print("-" * 42) print(" UP=Forward DOWN=Backward LEFT=TurnL RIGHT=TurnR") print(" G=RotateL I=RotateR SPACE=Stop") print(" A=AutoFL D=AutoFR Z=AutoBL X=AutoBR") print("-" * 42) print(" 1=Slow(60) 2=Medium(130) 3=Fast(200)") print(" ESC = Quit safely") print("=" * 42) print("READY — use arrow keys to drive!\n") # ── STATE ──────────────────────────────────────────────────────── last_cmd = '' # Last sent command — avoids duplicate sends last_speed = '3' # Default speed level: Fast CMD_NAMES = { 'F':'FORWARD', 'B':'BACKWARD', 'L':'TURN LEFT', 'R':'TURN RIGHT', 'S':'STOP', 'G':'ROTATE LEFT', 'I':'ROTATE RIGHT', 'A':'AUTO: Fwd+Left', 'D':'AUTO: Fwd+Right', 'Z':'AUTO: Bwd+Left', 'X':'AUTO: Bwd+Right', '1':'SPEED -> SLOW (60)', '2':'SPEED -> MEDIUM (130)', '3':'SPEED -> FAST (200)', } # ── SEND HELPERS ───────────────────────────────────────────────── def send(cmd): """Send only when command changes — prevents serial flooding.""" global last_cmd if cmd != last_cmd: arduino.write(cmd.encode()) last_cmd = cmd print(f">> {CMD_NAMES.get(cmd, cmd)}") def send_speed(lvl): """Send speed change and reset last_cmd so next move resends.""" global last_speed, last_cmd if lvl != last_speed: arduino.write(lvl.encode()) last_speed = lvl last_cmd = '' # Force resend of movement at new speed print(f">> {CMD_NAMES.get(lvl, lvl)}") # ── MAIN LOOP ───────────────────────────────────────────────────── # Runs ~12 times per second (80ms sleep) # Checks key state each iteration — hold key = continuous send # No key held = sends Stop to halt car try: while True: # Speed keys: 1 / 2 / 3 if keyboard.is_pressed('1'): send_speed('1') elif keyboard.is_pressed('2'): send_speed('2') elif keyboard.is_pressed('3'): send_speed('3') # Movement keys: arrow keys + special commands if keyboard.is_pressed('up'): send('F') # Forward elif keyboard.is_pressed('down'): send('B') # Backward elif keyboard.is_pressed('left'): send('L') # Turn Left elif keyboard.is_pressed('right'): send('R') # Turn Right elif keyboard.is_pressed('space'): send('S') # Stop elif keyboard.is_pressed('g'): send('G') # Rotate Left elif keyboard.is_pressed('i'): send('I') # Rotate Right elif keyboard.is_pressed('a'): send('A') # Auto Fwd+Left elif keyboard.is_pressed('d'): send('D') # Auto Fwd+Right elif keyboard.is_pressed('z'): send('Z') # Auto Bwd+Left elif keyboard.is_pressed('x'): send('X') # Auto Bwd+Right elif keyboard.is_pressed('esc'): break # Exit else: send('S') # No key = Stop time.sleep(0.08) # 80ms polling interval except KeyboardInterrupt: print("\nCtrl+C detected.") except Exception as e: print(f"\nError: {e}") # ── CLEANUP ────────────────────────────────────────────────────── print("Stopping car and closing port...") arduino.write(b'S') # Always stop before closing arduino.close() print("Done. Goodbye!")

## 10.1  Python Code — Key Sections Explained

| Code Section | What It Does | Why It Matters |
| --- | --- | --- |
| import serial, keyboard | Imports pyserial and keyboard libraries | Must be installed first: pip install pyserial keyboard |
| COM_PORT = "COM5" | Sets which serial port to connect to | Must match your Arduino port — check Arduino IDE Tools > Port |
| serial.Serial(COM_PORT, 9600) | Opens serial connection to Arduino | timeout=0.1 means read operations don't block forever |
| time.sleep(2) | Waits 2 seconds after opening port | Arduino resets when serial opens — waits for it to finish booting |
| last_cmd tracking | Stores the last command sent | Prevents sending the same command 12× per second (serial flood) |
| def send(cmd) | Sends command only if it changed | Efficient — only transmits on state change, not every loop tick |
| def send_speed(lvl) | Sends speed and resets last_cmd | Resets last_cmd so next movement resends at the new speed |
| keyboard.is_pressed("up") | Polls whether UP arrow is held down now | Returns True while held — unlike events which fire only once |
| else: send("S") | No key held = send Stop every loop | Guarantees car stops the instant ALL keys are released |
| time.sleep(0.08) | 80ms pause = ~12 checks/second | Smooth control without flooding the serial buffer |
| arduino.write(b"S") cleanup | Final Stop before port closes | Car ALWAYS stops safely even if script crashes or force-quit |

# Section 11 — Quick Reference Card

## 11.1  All Commands

| Key | Action | Left Motor | Right Motor | Available In |
| --- | --- | --- | --- | --- |
| F / ↑ Arrow | Move Forward | FORWARD | FORWARD | All 3 methods |
| B / ↓ Arrow | Move Backward | BACKWARD | BACKWARD | All 3 methods |
| L / ← Arrow | Turn Left | STOP | FORWARD | All 3 methods |
| R / → Arrow | Turn Right | FORWARD | STOP | All 3 methods |
| S / SPACE | Stop All | STOP | STOP | All 3 methods |
| G | Rotate Left in place | BACKWARD | FORWARD | All 3 methods |
| I | Rotate Right in place | FORWARD | BACKWARD | All 3 methods |
| A | Auto: Forward + Left | Sequence | Sequence | Serial + Python |
| D | Auto: Forward + Right | Sequence | Sequence | Serial + Python |
| Z | Auto: Backward + Left | Sequence | Sequence | Serial + Python |
| X | Auto: Backward + Right | Sequence | Sequence | Serial + Python |
| 1 | Speed: Slow (60) | 60 | 60 | Serial + Python |
| 2 | Speed: Medium (130) | 130 | 130 | Serial + Python |
| 3 | Speed: Fast (200) | 190 | 200 | Serial + Python |
| ESC | Quit Python script | — | — | Python only |

## 11.2  First Power-On Checklist

- Complete all wiring. Verify polarity (+ and −) before connecting battery.

- Connect Arduino GND → L298N GND (black wire) — mandatory.

- ENA and ENB jumper caps present on L298N (or wired to pins 6 and 9).

- Connect Arduino to laptop via USB. Arduino PWR LED lights red.

- Arduino IDE: Tools → Board: Arduino UNO → Port: your COM port.

- Upload Arduino code. Wait for "Done uploading" status message.

- Open Serial Monitor at 9600 baud. Confirm "RC CAR READY" appears.

- Place car on flat floor with 1m clearance. NOT on a desk.

- Type F → Enter. Car moves forward. Type S → Enter. Car stops.

- Test all directions B/L/R/G/I. Test auto: A/D/Z/X.

- Any motor wrong direction? Swap its two wires at L298N output.

- Tune drift: lower L_SPEED if drifts right, lower R_SPEED if drifts left.

Project Status: Phases 1–4 COMPLETE Next milestone: Phase 5 — Bluetooth wireless control (HC-05 module, ~$5). Final goal: Phase 7 — Fully autonomous line-following robot. All three code files are complete, tested, and documented in this report.