SMART RC CAR

WITH GY-271 MAGNETOMETER COMPASS

Project Report

Bluetooth Control  |  USB Control  |  Digital Compass

Technologies Used

Arduino Uno  •  HC-05 Bluetooth  •  GY-271 QMC5883L  •  L298N Motor Driver  •  Python

# 1. Project Overview

This project implements a Smart RC Car controlled wirelessly via Bluetooth and via USB serial connection. The car integrates a GY-271 QMC5883L magnetometer/compass module that continuously broadcasts heading direction and azimuth angle, providing real-time orientation data to the Python controller on a PC.

## 1.1  Key Features

- Dual control modes: Bluetooth (HC-05) and USB Serial

- Real-time compass heading using GY-271 (QMC5883L)

- 8-direction compass support: N, NE, E, SE, S, SW, W, NW

- Variable speed control: Slow, Medium, Fast

- Live dashboard with compass rose in Python

- Smooth motor control via L298N motor driver

## 1.2  Components

| Component | Model / Value | Qty | Purpose |
| --- | --- | --- | --- |
| Microcontroller | Arduino Uno | 1 | Main controller |
| Magnetometer | GY-271 (QMC5883L) | 1 | Compass / heading |
| Bluetooth Module | HC-05 | 1 | Wireless control |
| Motor Driver | L298N | 1 | Drive DC motors |
| DC Motors | Generic TT Motor | 4 | Drive wheels |
| Power Supply | 7.4 V LiPo / 9 V | 1 | Power source |
| PC / Laptop | Windows with Python 3 | 1 | Controller UI |

# 2. Hardware Wiring

## 2.1  GY-271 Magnetometer → Arduino (I2C)

The GY-271 communicates via I²C. Connect it as follows:

| GY-271 Pin | Arduino Pin | Notes |
| --- | --- | --- |
| VCC | 3.3 V | Use 3.3 V only — 5 V will damage sensor |
| GND | GND | Common ground |
| SDA | A4 (SDA) | I²C data line |
| SCL | A5 (SCL) | I²C clock line |
| DRDY | — | Optional data-ready pin, leave unconnected |

⚠  Important: The QMC5883L is a 3.3 V device. Do NOT connect VCC to the 5 V Arduino pin.

## 2.2  HC-05 Bluetooth → Arduino (SoftwareSerial)

The HC-05 is wired to SoftwareSerial pins 10 (RX) and 11 (TX):

| HC-05 Pin | Arduino Pin | Notes |
| --- | --- | --- |
| VCC | 5 V | Module requires 5 V |
| GND | GND | Common ground |
| TX | Pin 10 (RX-SS) | HC-05 TX → Arduino software RX |
| RX | Pin 11 (TX-SS) | Arduino software TX → HC-05 RX |
| EN | — | Leave unconnected (normal mode) |
| STATE | — | Optional status LED |

Note: The HC-05 RX pin is 3.3 V tolerant on some modules; if yours is not, use a voltage divider (1 kΩ + 2 kΩ) on the Arduino TX line.

## 2.3  L298N Motor Driver → Arduino

| L298N Pin | Arduino Pin | Notes |
| --- | --- | --- |
| IN1 | Pin 2 | Left motor direction A |
| IN2 | Pin 3 | Left motor direction B |
| IN3 | Pin 4 | Right motor direction A |
| IN4 | Pin 5 | Right motor direction B |
| ENA | Pin 6 (PWM) | Left motor speed (analogWrite) |
| ENB | Pin 9 (PWM) | Right motor speed (analogWrite) |
| VCC | Battery + | Motor supply voltage (7–12 V) |
| GND | Battery – / Arduino GND | Common ground |
| +5V | Arduino 5V (optional) | If on-board 5V regulator used |

## 2.4  Full Wiring Summary

| Signal | From | To | Wire Color (suggestion) |
| --- | --- | --- | --- |
| I²C SDA | GY-271 SDA | Arduino A4 | Yellow |
| I²C SCL | GY-271 SCL | Arduino A5 | Green |
| 3.3 V | Arduino 3V3 | GY-271 VCC | Red |
| GND | Arduino GND | GY-271 GND | Black |
| BT TX→RX | HC-05 TX | Arduino Pin 10 | White |
| BT RX←TX | Arduino P11 | HC-05 RX | Blue |
| 5 V | Arduino 5V | HC-05 VCC | Red |
| Motor IN1–4 | Arduino 2–5 | L298N IN1–4 | Orange |
| Motor ENA | Arduino 6 | L298N ENA | Purple |
| Motor ENB | Arduino 9 | L298N ENB | Brown |

# 3. Bluetooth Control

In Bluetooth mode, the PC connects to the HC-05 via a virtual COM port (e.g., COM7 or COM8). The Python script sends single-character commands and reads compass data — all through the same wireless serial link.

## 3.1  Arduino Code — Bluetooth + Compass

Upload this sketch to the Arduino. It listens for commands on both the HC-05 SoftwareSerial port and the USB serial, drives the motors, reads the GY-271, and broadcasts the heading back over Bluetooth.

#include <SoftwareSerial.h>

#include <QMC5883LCompass.h>

// Bluetooth on pins 10 (RX) / 11 (TX)

SoftwareSerial bluetooth(10, 11);

QMC5883LCompass compass;

// L298N motor pins

#define IN1 2

#define IN2 3

#define IN3 4

#define IN4 5

#define ENA 6

#define ENB 9

int L_SPEED    = 220;

int R_SPEED    = 220;

int TURN_SPEED = 170;

void setup() {

pinMode(IN1,OUTPUT); pinMode(IN2,OUTPUT);

pinMode(IN3,OUTPUT); pinMode(IN4,OUTPUT);

pinMode(ENA,OUTPUT); pinMode(ENB,OUTPUT);

Serial.begin(9600);

bluetooth.begin(9600);

compass.init();

stopCar();

Serial.println("SMART RC CAR READY");

bluetooth.println("SMART RC CAR READY");

}

void loop() {

char cmd = 0;

if (bluetooth.available()) cmd = bluetooth.read();

if (Serial.available())   cmd = Serial.read();

if (cmd) control(cmd);

// --- Compass reading ---

compass.read();

int angle = compass.getAzimuth();

String dir;

if (angle < 45  || angle >= 315) dir = "NORTH";

else if (angle < 135)             dir = "EAST";

else if (angle < 225)             dir = "SOUTH";

else                              dir = "WEST";

// Broadcast to both USB and Bluetooth

Serial.print("ANGLE: ");   Serial.print(angle);

Serial.print("  ");        Serial.println(dir);

bluetooth.print("ANGLE: "); bluetooth.print(angle);

bluetooth.print("  ");      bluetooth.println(dir);

delay(300);

}

void control(char c) {

switch(c) {

case 'F': case 'f': forwardCar();  break;

case 'B': case 'b': backCar();     break;

case 'L': case 'l': leftCar();     break;

case 'R': case 'r': rightCar();    break;

case 'G': case 'g': rotateLeft();  break;

case 'I': case 'i': rotateRight(); break;

case 'S': case 's': stopCar();     break;

case '1': L_SPEED = R_SPEED =  80; break;

case '2': L_SPEED = R_SPEED = 150; break;

case '3': L_SPEED = R_SPEED = 220; break;

}

}

void forwardCar()  { digitalWrite(IN1,HIGH); digitalWrite(IN2,LOW);

digitalWrite(IN3,HIGH); digitalWrite(IN4,LOW);

analogWrite(ENA,L_SPEED); analogWrite(ENB,R_SPEED); }

void backCar()     { digitalWrite(IN1,LOW);  digitalWrite(IN2,HIGH);

digitalWrite(IN3,LOW);  digitalWrite(IN4,HIGH);

analogWrite(ENA,L_SPEED); analogWrite(ENB,R_SPEED); }

void leftCar()     { digitalWrite(IN1,LOW);  digitalWrite(IN2,LOW);

digitalWrite(IN3,HIGH); digitalWrite(IN4,LOW);

analogWrite(ENA,0); analogWrite(ENB,TURN_SPEED); }

void rightCar()    { digitalWrite(IN1,HIGH); digitalWrite(IN2,LOW);

digitalWrite(IN3,LOW);  digitalWrite(IN4,LOW);

analogWrite(ENA,TURN_SPEED); analogWrite(ENB,0); }

void rotateLeft()  { digitalWrite(IN1,LOW);  digitalWrite(IN2,HIGH);

digitalWrite(IN3,HIGH); digitalWrite(IN4,LOW);

analogWrite(ENA,TURN_SPEED); analogWrite(ENB,TURN_SPEED); }

void rotateRight() { digitalWrite(IN1,HIGH); digitalWrite(IN2,LOW);

digitalWrite(IN3,LOW);  digitalWrite(IN4,HIGH);

analogWrite(ENA,TURN_SPEED); analogWrite(ENB,TURN_SPEED); }

void stopCar()     { digitalWrite(IN1,LOW);  digitalWrite(IN2,LOW);

digitalWrite(IN3,LOW);  digitalWrite(IN4,LOW);

analogWrite(ENA,0); analogWrite(ENB,0); }

## 3.2  Python Bluetooth Controller

Install dependencies first:  pip install pyserial keyboard

import serial, keyboard, time, sys

# -------- SETTINGS --------

COM_PORT = "COM8"   # HC-05 Bluetooth virtual COM port

BAUD     = 9600

# -------- CONNECT --------

print("Connecting via Bluetooth...")

try:

bt = serial.Serial(COM_PORT, BAUD, timeout=0.1)

time.sleep(2)

print("Bluetooth Connected — SMART RC CAR STARTED\n")

except:

print("Bluetooth not connected. Check COM port.")

sys.exit()

print("""

========= CONTROL =========

UP    : Forward    DOWN  : Backward

LEFT  : Left       RIGHT : Right

G     : Rotate L   I     : Rotate R

1/2/3 : Slow/Med/Fast     SPACE : Stop

ESC   : Exit

===========================""")

last_cmd = ""

def send(cmd):

global last_cmd

if cmd != last_cmd:

bt.write(cmd.encode())

print("CMD:", cmd)

last_cmd = cmd

while True:

# ---- Read compass over Bluetooth ----

while bt.in_waiting:

data = bt.readline().decode(errors="ignore").strip()

if "ANGLE" in data:

print("Compass:", data)

# ---- Keyboard control ----

if   keyboard.is_pressed("up"):    send("F")

elif keyboard.is_pressed("down"):  send("B")

elif keyboard.is_pressed("left"):  send("L")

elif keyboard.is_pressed("right"): send("R")

elif keyboard.is_pressed("g"):     send("G")

elif keyboard.is_pressed("i"):     send("I")

elif keyboard.is_pressed("1"):     send("1")

elif keyboard.is_pressed("2"):     send("2")

elif keyboard.is_pressed("3"):     send("3")

elif keyboard.is_pressed("space"): send("S")

elif keyboard.is_pressed("esc"):   send("S"); break

else:                               send("S")

time.sleep(0.05)

bt.close()

print("Car stopped.")

## 3.3  Bluetooth Command Reference

| Key | Command Sent | Action |
| --- | --- | --- |
| ↑ Up Arrow | F | Forward |
| ↓ Down Arrow | B | Backward |
| ← Left Arrow | L | Turn Left |
| → Right Arrow | R | Turn Right |
| G | G | Rotate Left (tank) |
| I | I | Rotate Right (tank) |
| 1 | 1 | Speed: Slow (PWM 80) |
| 2 | 2 | Speed: Medium (PWM 150) |
| 3 | 3 | Speed: Fast (PWM 220) |
| Space | S | Stop |
| ESC | S → Exit | Stop & quit |

# 4. USB (Serial) Control

In USB mode, the Arduino is connected directly to the PC via the USB cable (no Bluetooth required). The Python script opens the Arduino's COM port, sends movement commands, and receives compass data over the same USB link. This is ideal for bench testing or indoor environments.

## 4.1  Arduino Code — USB + Compass

This sketch accepts commands through the native USB serial port and outputs 8-direction compass data.

#include <Wire.h>

#include <QMC5883LCompass.h>

QMC5883LCompass compass;

// L298N motor pins

#define IN1 2

#define IN2 3

#define IN3 4

#define IN4 5

#define ENA 6

#define ENB 9

int speedCar = 180;

void setup() {

pinMode(IN1,OUTPUT); pinMode(IN2,OUTPUT);

pinMode(IN3,OUTPUT); pinMode(IN4,OUTPUT);

pinMode(ENA,OUTPUT); pinMode(ENB,OUTPUT);

Serial.begin(9600);

Wire.begin();

compass.init();

stopCar();

Serial.println("USB RC CAR + COMPASS READY");

}

void loop() {

// --- USB command ---

if (Serial.available()) {

char cmd = Serial.read();

if (cmd=='F') forward();

if (cmd=='B') backward();

if (cmd=='L') left();

if (cmd=='R') right();

if (cmd=='S') stopCar();

if (cmd=='1') speedCar =  80;

if (cmd=='2') speedCar = 150;

if (cmd=='3') speedCar = 220;

}

// --- 8-direction compass reading ---

compass.read();

int x     = compass.getX();

int y     = compass.getY();

int angle = atan2(y, x) * 180 / PI;

if (angle < 0) angle += 360;

String direction;

if      (angle >= 337 || angle <  22) direction = "NORTH";

else if (angle <  67)                 direction = "NORTH EAST";

else if (angle < 112)                 direction = "EAST";

else if (angle < 157)                 direction = "SOUTH EAST";

else if (angle < 202)                 direction = "SOUTH";

else if (angle < 247)                 direction = "SOUTH WEST";

else if (angle < 292)                 direction = "WEST";

else                                  direction = "NORTH WEST";

Serial.print("ANGLE:");

Serial.print(angle);

Serial.print("  DIR:");

Serial.println(direction);

delay(200);

}

void forward()  { digitalWrite(IN1,HIGH); digitalWrite(IN2,LOW);

digitalWrite(IN3,HIGH); digitalWrite(IN4,LOW);

analogWrite(ENA,speedCar); analogWrite(ENB,speedCar); }

void backward() { digitalWrite(IN1,LOW);  digitalWrite(IN2,HIGH);

digitalWrite(IN3,LOW);  digitalWrite(IN4,HIGH);

analogWrite(ENA,speedCar); analogWrite(ENB,speedCar); }

void left()     { digitalWrite(IN1,LOW);  digitalWrite(IN2,LOW);

digitalWrite(IN3,HIGH); digitalWrite(IN4,LOW);

analogWrite(ENA,0); analogWrite(ENB,speedCar); }

void right()    { digitalWrite(IN1,HIGH); digitalWrite(IN2,LOW);

digitalWrite(IN3,LOW);  digitalWrite(IN4,LOW);

analogWrite(ENA,speedCar); analogWrite(ENB,0); }

void stopCar()  { digitalWrite(IN1,LOW);  digitalWrite(IN2,LOW);

digitalWrite(IN3,LOW);  digitalWrite(IN4,LOW);

analogWrite(ENA,0); analogWrite(ENB,0); }

## 4.2  Python USB Controller

import serial, keyboard, time

PORT = "COM5"      # Arduino USB COM port

BAUD = 9600

arduino = serial.Serial(PORT, BAUD, timeout=0.1)

time.sleep(2)

print("RC CAR USB CONTROL + COMPASS")

print("Arrow keys to drive | ESC to exit")

last_cmd = ""

def send(cmd):

global last_cmd

if cmd != last_cmd:

arduino.write(cmd.encode())

last_cmd = cmd

print("CMD:", cmd)

while True:

# ---- Read compass data ----

if arduino.in_waiting:

data = arduino.readline().decode(errors="ignore").strip()

if data:

print(data)

# ---- Key control ----

if   keyboard.is_pressed("up"):    send("F")

elif keyboard.is_pressed("down"):  send("B")

elif keyboard.is_pressed("left"):  send("L")

elif keyboard.is_pressed("right"): send("R")

elif keyboard.is_pressed("1"):     send("1")

elif keyboard.is_pressed("2"):     send("2")

elif keyboard.is_pressed("3"):     send("3")

elif keyboard.is_pressed("space"): send("S")

elif keyboard.is_pressed("esc"):   send("S"); break

else:                               send("S")

time.sleep(0.08)

arduino.close()

print("CAR STOPPED")

## 4.3  Dual-Port Mode (BT + USB simultaneously)

For maximum flexibility, the combined sketch (Section 3.1) already accepts commands from both the Bluetooth SoftwareSerial port AND the USB serial port simultaneously. The Python dual-port script below opens two COM ports: BT for driving, USB for reading compass data with higher precision.

import serial, keyboard, time, threading, os, sys

BT_PORT  = "COM8"   # HC-05 Bluetooth

USB_PORT = "COM5"   # Arduino USB

BAUD     = 9600

# Connect Bluetooth

try:

bt = serial.Serial(BT_PORT, BAUD, timeout=0.1)

time.sleep(2)

print("Bluetooth OK:", BT_PORT)

except Exception as e:

print("BT Error:", e); sys.exit(1)

# Connect USB (optional — compass only)

usb, usb_ok = None, False

try:

usb = serial.Serial(USB_PORT, BAUD, timeout=1)

time.sleep(2)

usb_ok = True

print("USB OK:", USB_PORT)

except:

print("USB not connected — compass disabled")

compass_angle = "---"

compass_dir   = "---"

last_cmd      = ""

running       = True

def read_compass():

global compass_angle, compass_dir

while running:

try:

if usb_ok and usb.in_waiting > 0:

line = usb.readline().decode('utf-8', errors='ignore').strip()

if line.startswith("ANGLE:"):

parts = line.replace("ANGLE:","").strip().split()

if len(parts) >= 2:

compass_angle = parts[0] + " deg"

compass_dir   = parts[1]

except: pass

time.sleep(0.05)

threading.Thread(target=read_compass, daemon=True).start()

def send(cmd):

global last_cmd

if cmd != last_cmd:

bt.write(cmd.encode())

last_cmd = cmd

status = "STOPPED"

tick   = 0

try:

while True:

if   keyboard.is_pressed('up'):    send('F'); status = "FORWARD"

elif keyboard.is_pressed('down'):  send('B'); status = "BACKWARD"

elif keyboard.is_pressed('left'):  send('L'); status = "TURN LEFT"

elif keyboard.is_pressed('right'): send('R'); status = "TURN RIGHT"

elif keyboard.is_pressed('g'):     send('G'); status = "ROTATE LEFT"

elif keyboard.is_pressed('i'):     send('I'); status = "ROTATE RIGHT"

elif keyboard.is_pressed('space'): send('S'); status = "STOPPED"

elif keyboard.is_pressed('1'):     send('1'); status = "SLOW"

elif keyboard.is_pressed('2'):     send('2'); status = "MEDIUM"

elif keyboard.is_pressed('3'):     send('3'); status = "FAST"

elif keyboard.is_pressed('esc'):   send('S'); break

else:                              send('S'); status = "STOPPED"

tick += 1

if tick % 5 == 0:

os.system('cls')

print(f"Status: {status}")

print(f"Angle : {compass_angle}")

print(f"Dir   : {compass_dir}")

time.sleep(0.08)

except Exception as e:

print("Error:", e)

running = False

bt.write(b'S')

bt.close()

if usb_ok: usb.close()

print("Car stopped. Goodbye!")

# 5. Digital Compass — Standalone Code

This section provides the complete standalone compass sketch for the GY-271 (QMC5883L). Use it to verify the sensor works correctly before integrating it into the full RC car project.

## 5.1  Required Arduino Library

Install via Arduino IDE → Library Manager → search "QMC5883LCompass" by MPrograms.

| Library | QMC5883LCompass by MPrograms — available in Arduino Library Manager |
| --- | --- |

## 5.2  Standalone Compass Arduino Sketch

This sketch reads azimuth (0–360°) and 8-point cardinal direction with smoothing enabled. Upload and open Serial Monitor at 9600 baud.

#include <Wire.h>

#include <QMC5883LCompass.h>

QMC5883LCompass compass;

void setup() {

Serial.begin(9600);

compass.init();

// Enable smoothing over 10 samples,

// true = 8 directions (N, NE, E, SE, S, SW, W, NW)

compass.setSmoothing(10, true);

}

void loop() {

// Read the sensor

compass.read();

// Get azimuth angle (0 – 360 degrees)

int azimuth = compass.getAzimuth();

// Get 2-character direction string (e.g., "N", "NE", "E")

char direction[3];

compass.getDirection(direction, azimuth);

// Print to Serial Monitor

Serial.print("Azimuth: ");

Serial.print(azimuth);

Serial.print(" deg  |  Direction: ");

Serial.println(direction);

delay(500);  // 0.5 s between readings

}

## 5.3  Example Serial Monitor Output

Azimuth:   9 deg  |  Direction: N

Azimuth:  43 deg  |  Direction: NE

Azimuth:  91 deg  |  Direction: E

Azimuth: 179 deg  |  Direction: S

Azimuth: 273 deg  |  Direction: W

## 5.4  Direction Angle Chart

| Direction | Abbreviation | Angle Range (°) |
| --- | --- | --- |
| North | N | 338 – 360 and 0 – 22 |
| North-East | NE | 23 – 67 |
| East | E | 68 – 112 |
| South-East | SE | 113 – 157 |
| South | S | 158 – 202 |
| South-West | SW | 203 – 247 |
| West | W | 248 – 292 |
| North-West | NW | 293 – 337 |

## 5.5  Calibration Tips

- After power-on, slowly rotate the sensor through all axes (figure-8 motion) for 10–15 seconds to allow the QMC5883L to auto-calibrate.

- Keep metal objects, motors, and high-current wires at least 5 cm from the sensor to avoid magnetic interference.

- Mount the GY-271 horizontally and level for most accurate readings.

- The setSmoothing(10, true) call averages the last 10 readings; increase the number (max 10) to reduce jitter.

# 6. Troubleshooting

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| Compass reads 0 always | GY-271 VCC on 5V | Move to 3.3 V pin |
| '0x00 not found' error | Wrong I²C address | Run I²C scanner sketch |
| Bluetooth won't pair | Default PIN not set | Default PIN: 1234 or 0000 |
| COM port busy error | Arduino IDE Monitor open | Close Serial Monitor first |
| Car moves wrong way | Motor polarity swapped | Swap IN1/IN2 or IN3/IN4 |
| Noisy compass near motors | EMI from motor wires | Move GY-271, add decoupling |